Instruction:

Complete all questions in one hour.

- Which one is the characteristic of Harvard Architecture?

- Program  and Data stored in Separate Memory

- Program and Data stored in same Memory

- Program and data stored in Cache Memory

- All of Above

- Which of the following is the working cycle of CPU?

- Decode, Execute, and Fetch

- Fetch, Decode, and execute

- Fetch, Execute, and Decode

- All of Above

- Any condition that causes a processor to stall is called as _________

- Hazard

- Page fault

- System error

- None of the mentioned

- What does the control unit generate to control other units?

  - Transfer signals

  - Command Signal

  - Control signals

  - Timing signals

- What must the processors of all computers have?

  - Control unit

  - ALU

  - Register

  - All of these

- Which is the fastest memory in the computer?

- Cache

- RAM

- Register

- Hard disk

- With the help of _______ we reduce the memory access time:

  - SDRAM

  - Cache

  - Heaps

  - Higher capacity RAMs

- For a given FINITE number of instructions to be executed, which architecture of the processor provides for faster execution?

  - ISA

  - ANSA

  - Super-scalar

  - All of the mentioned

- A processor performing fetch or decoding of different instruction during the execution of another instruction is called ______

  - Super-scaling

  - Pipe-lining

  - Parallel Computation

  - None of the mentioned

- A 24 bit address generates an address space of ______ locations.

  - 1024

  - 4096

  - 248

  - 16,777,216

- The USA contains about 3100 counties. Suppose you have a table that stores, for each county, its name (up to 40 characters in 8-bit ASCII), its state (a two-letter code), its population, and its median income (both as 32-bit numbers). How much space would the whole database take in memory?

- The computer has a maximum addressable memory of 16 Gigabytes. Its address bus width is 32.

- Calculate the width of the data bus.

- State the effect that adding one new line to the address bus would have on the maximum addressable memory.

- Explain the Instruction cycle of the processor by giving an example of the program

Load: [10]

Add : [11]

Store:  [12]

- Write short notes on the following topic:

- Von Neumann and Harvard Architecture

- RISC vs CISC architecture